ESP32 Co-Processor

STM32 OTA Firmware Updater

PlatformIO + ESP-IDF | XModem Protocol

# Overview

This project enables an ESP32 to act as a co-processor that remotely updates the firmware of an STM32 microcontroller via XModem protocol over UART. The ESP32 opens a Wi-Fi Access Point and serves a web interface for uploading the STM32 .bin file, then handles the full bootloader entry sequence and XModem transfer automatically.

## System Architecture

| Component | Role |
| --- | --- |
| ESP32 | Wi-Fi AP + HTTP server + XModem transmitter |
| STM32 (target) | Custom XModem bootloader receiver |
| PC / Phone | Connects to ESP32 AP, uploads .bin via browser |
| SPIFFS | Temporary storage for firmware binary on ESP32 |

# Hardware Connections

| ESP32 GPIO | STM32 Pin | Function | Notes |
| --- | --- | --- | --- |
| GPIO 32 | PA0 (BOOT0) | Bootloader trigger | HIGH = enter bootloader |
| GPIO 33 | NRST | Reset | Active LOW pulse |
| GPIO 17 (TX) | PA10 USART1 RX | UART data | 115200 baud |
| GPIO 16 (RX) | PA9 USART1 TX | UART data | 115200 baud |
| GND | GND | Common ground | Required |

Warning: Do NOT use GPIO 25/26 — these are DAC1/DAC2 on ESP32 and will cause resets during boot.

GPIO 34, 35, 36, 39 are input-only and cannot be used for output signals.

# Project Structure

stm32_ota/

├── platformio.ini          # PlatformIO config

├── partitions.csv          # Custom flash partition table

├── sdkconfig.esp32dev      # ESP-IDF SDK config

└── src/

├── main.c              # App main: AP + HTTP server

├── stm32_updater.c     # XModem transmitter + GPIO control

└── stm32_updater.h     # Public API

# Configuration

## platformio.ini

[env:esp32dev]

platform    = espressif32

board       = esp32dev

framework   = espidf

board_build.partitions = partitions.csv

monitor_speed = 115200

## partitions.csv

| Name | Type | SubType | Offset | Size |
| --- | --- | --- | --- | --- |
| nvs | data | nvs | 0x9000 | 0x6000 |
| phy_init | data | phy | 0xF000 | 0x1000 |
| factory | app | factory | 0x10000 | 0x180000 |
| spiffs | data | spiffs | 0x190000 | 0x60000 |

SPIFFS partition is 384 KB — sufficient for STM32 firmware binaries up to 256 KB.

## Wi-Fi Access Point

| Parameter | Value |
| --- | --- |
| SSID | ESP32-STM32-OTA |
| Password | 12345678 |
| IP Address | 192.168.4.1 |
| Security | WPA2-PSK |

## Pin Definitions (stm32_updater.h)

#define STM32_BOOT_PIN      GPIO_NUM_32   // PA0 bootloader trigger

#define STM32_NRST_PIN      GPIO_NUM_33   // NRST reset

#define STM32_UART_PORT     UART_NUM_2

#define STM32_UART_TX       GPIO_NUM_17

#define STM32_UART_RX       GPIO_NUM_16

#define STM32_UART_BAUD     115200

## Timing Parameters

| Define | Value | Description |
| --- | --- | --- |
| RESET_ASSERT_MS | 20 ms | NRST low pulse width |
| BOOT_SETUP_MS | 10 ms | PA0 setup time before reset |
| BOOTLOADER_READY_MS | 3500 ms | Wait for STM32 bootloader NAK_INIT_DELAY (3000 ms) + margin |
| XMODEM_TIMEOUT_MS | 3000 ms | Per-byte receive timeout (matches STM32 side) |
| XMODEM_MAX_RETRY | 10 | Max block retransmit attempts |

# Operation

## Bootloader Entry Sequence

The ESP32 enters the STM32 custom bootloader using the following hardware sequence:

1. Assert PA0 (BOOT0) HIGH   → GPIO32 = 1

2. Assert NRST LOW           → GPIO33 = 0  (20 ms)

3. Release NRST HIGH         → GPIO33 = 1

4. Wait 3500 ms              → STM32 bootloader sends NAK

5. Flush UART RX buffer      → discard any garbage bytes

6. Begin XModem handshake

## XModem Transfer

- Protocol: XModem with 8-bit checksum (classic mode)

- Block size: 128 bytes

- Padding: last block padded with 0x1A (SUB character)

- Block numbers wrap: 0xFF -> 0x00 automatically (uint8_t)

- File is streamed from SPIFFS — no large RAM buffer required

## Bootloader Exit Sequence

1. De-assert PA0 (BOOT0)     → GPIO32 = 1 (already high, no change)

2. Assert NRST LOW           → GPIO33 = 0  (20 ms)

3. Release NRST HIGH         → GPIO33 = 1

STM32 boots into application

## Web Upload Flow

PC/Phone  ──Wi-Fi──►  ESP32 AP (192.168.4.1)

│

HTTP POST /upload (.bin file)

│

Save to SPIFFS /spiffs/stm32fw.bin

│

stm32_enter_bootloader()

│

XModem stream SPIFFS → STM32 UART

│

stm32_exit_bootloader()

│

HTTP response: 'Flash success!'

# Usage

## Build and Flash ESP32

# Build

pio run

# Flash ESP32 firmware

pio run --target upload

# Monitor serial output

pio device monitor

## Update STM32 Firmware

- Power on ESP32

- Connect PC or phone to Wi-Fi: "ESP32-STM32-OTA" / password: "12345678"

- Open browser and navigate to http://192.168.4.1

- Select the STM32 .bin file and click Upload & Flash STM32

- Wait 10-30 seconds for transfer to complete

- Browser shows "Flash success!" — STM32 reboots into new firmware

# Memory Optimization

The firmware binary is never loaded entirely into RAM. SPIFFS streaming keeps memory usage minimal:

| Buffer | Size | Location |
| --- | --- | --- |
| HTTP chunk buffer | 512 bytes | Stack (temporary) |
| XModem block buffer | 128 bytes | Stack (per block) |
| XModem packet | 131 bytes | Stack (per block) |
| SPIFFS file handle | ~100 bytes | Heap |
| Firmware binary (old) | 256 KB static | Removed — was cause of DRAM overflow |

# STM32 Custom Bootloader Requirements

The STM32 must be running a custom bootloader (not ST system bootloader) with the following characteristics:

| Parameter | Value |
| --- | --- |
| UART peripheral | USART1 (huart1) |
| Baud rate | 115200 |
| XModem mode | Checksum (classic) — sends NAK 0x15 |
| NAK_INIT_DELAY | 3000 ms after reset |
| XMODEM_TIMEOUT_MS | 3000 ms per byte |
| XMODEM_MAX_RETRIES | 3 |
| APP_START_ADDR | 0x08004000 (Sector 1) |
| BOOT0 trigger | PA0 = HIGH at reset |

# Troubleshooting

| Symptom | Cause | Fix |
| --- | --- | --- |
| ESP32 resets in boot loop | GPIO 25/26 conflict with DAC | Use GPIO 32/33 instead |
| DRAM overflow at link | 256 KB static fw_buf in BSS | Stream from SPIFFS, no static buffer |
| Garbage handshake bytes | BOOTLOADER_READY_MS too short | Set to 3500 ms (>= NAK_INIT_DELAY) |
| Block 1 always NAK/CAN | TX/RX lines swapped | Verify ESP TX -> STM32 RX wiring |
| Timeout on all handshake bytes | Wrong baud rate or UART port | Check 115200 baud, UART_NUM_2 |
| IllegalInstruction panic | uart_driver_install called twice | Guard with uart_is_driver_installed() |

# API Reference

## stm32_updater.h

| Function | Description |
| --- | --- |
| void stm32_hw_init(void) | Initialize GPIO (BOOT0, NRST) and UART2. Call once in app_main(). |
| esp_err_t stm32_enter_bootloader(void) | Assert BOOT0, pulse NRST, wait for bootloader ready. |
| esp_err_t stm32_exit_bootloader(void) | De-assert BOOT0, pulse NRST to reboot into application. |
| esp_err_t stm32_ota_from_spiffs(const char *path) | Full OTA sequence: enter bootloader, XModem stream from file, exit bootloader. |

# Dependencies

- ESP-IDF 5.x (via PlatformIO espressif32 platform)

- esp_wifi — Wi-Fi Access Point

- esp_http_server — HTTP upload server

- esp_spiffs / esp_vfs_spiffs — Filesystem for firmware storage

- driver/uart — UART2 for STM32 communication

- driver/gpio — BOOT0 and NRST control